# Thermostat Lab

Zachary N. Locke

Southern New Hampshire University

CS-350: Emerging Sys Arch & Tech

Roland Morales

February 22, 2026

Temperature Sensor Integration Reflection

For this particular project, I developed a smart thermostat prototype on a Raspberry Pi 4B board using Python. The circuit consists of an AHT20 temperature and humidity sensor connected through I²C communication, a 16×2 character LCD display, three pushbuttons, and two LEDs: one red and one blue. The combination of these peripherals enables the thermostat to sense the temperature in the room, show the status of the system to the user, take input to change the set point and mode, and show the operation of the heating or cooling system using the LEDs.

The software is based on the machine with three states: OFF, HEAT, and COOL. The User can switch between these modes with two buttons. There are buttons to set the temperature, allowing the User to decrease it by one degree at a time. Every cycle, the software reads data from the AHT20 sensor. The device then compares the current temperature with the preset value and adjusts the LEDs accordingly. If the temperature rises too high, the heating LED will dim. Conversely, when the temperature falls too low, the cooling LED will dim. Once the temperature matches the set value, the corresponding LED lights up. The LCD displays the date/time, the temperature mode in use, and the set value. Every 30 seconds, a data string is transmitted through the UART to the analysis server of SysTecs.

For production purposes, SysTec requires a Wi-Fi-enabled thermostat with I²C interface, GPIO interface, UART interface, and LCD interface. Raspberry Pi architecture suits these requirements and provides ample memory and flash space; however, its power consumption and price are too high for a thermostat. A Microchip microcontroller design would provide lower cost and power while still supporting I²C, GPIO, and UART; paired with a Microchip Wi-Fi module, it can meet the cloud-connectivity requirement with enough memory for this small codebase. A Freescale/NXP Cortex-M architecture offers similar advantages and strong IoT support, but Microchip’s mature thermostat and HVAC reference designs make it a better fit here.